CSE 2109 – Electronic Engineering

50 Most Important MCQs – BJT, Transistor, FET, JFET & MOSFET

Selected from the uploaded class materials | Answer is given immediately below each question

⚠ Exam Focus: These are high-priority questions based on the uploaded notes. They are selected for exam preparation, but no question can be guaranteed to appear exactly in the exam.

BJT / Transistor Basics

1. What does BJT stand for?

A) Bipolar Junction Transistor

B) Binary Junction Transistor

C) Base Junction Technology

D) Bipolar Joint Terminal

Answer: A

2. How many terminals does a BJT have?

A) 2

B) 3

C) 4

D) 5

Answer: B

3. Which three terminals belong to a BJT?

A) Gate, Source, Drain

B) Anode, Cathode, Gate

C) Emitter, Base, Collector

D) Input, Output, Ground

Answer: C

4. A transistor consists essentially of how many pn junctions?

A) One

B) Two

C) Three

D) Four

Answer: B

5. A transistor can be regarded as a combination of:

A) Two diodes connected back to back

B) Two resistors in series

C) Two capacitors

D) One diode and one capacitor

Answer: A

6. Which region of a BJT is very thin and lightly doped?

A) Emitter

B) Base

C) Collector

D) Substrate

Answer: B

7. Which BJT region is heavily doped to inject a large number of charge carriers?

A) Base

B) Collector

C) Emitter

D) Junction

Answer: C

8. Which BJT region is moderately doped and physically larger to dissipate heat?

A) Base

B) Collector

C) Emitter

D) Gate

Answer: B

9. What are the main charge carriers in an NPN transistor?

A) Holes

B) Electrons

C) Ions

D) Photons

Answer: B

10. What are the main charge carriers in a PNP transistor?

A) Electrons

B) Holes

C) Neutrons

D) Ions

Answer: B

11. Which equation represents the transistor current relationship?

A) IE = IC − IB

B) IE = IB + IC

C) IC = IE + IB

D) IB = IE + IC

Answer: B

12. In normal transistor action, approximately what percentage of emitter-injected carriers cross into the collector according to the notes?

A) Less than 5%

B) About 25%

C) More than 95%

D) 50%

Answer: C

13. In the active region of a BJT, the emitter-base junction is ___ and collector-base junction is ___.

A) reverse biased, reverse biased

B) forward biased, reverse biased

C) reverse biased, forward biased

D) forward biased, forward biased

Answer: B

14. In the saturation region of a BJT, both junctions are:

A) Reverse biased

B) Forward biased

C) Open

D) Unbiased

Answer: B

15. In the cut-off region of a BJT, both junctions are:

A) Forward biased

B) Reverse biased

C) Shorted

D) Saturated

Answer: B

16. In which BJT region does the transistor normally act as an amplifier?

A) Cut-off

B) Saturation

C) Active

D) Breakdown

Answer: C

17. In which two BJT regions does the transistor act as a switch?

A) Active and breakdown

B) Cut-off and saturation

C) Active and saturation

D) Cut-off and active

Answer: B

18. At cut-off, the collector-emitter voltage is approximately:

A) 0 V

B) VCC

C) VBE

D) βVCC

Answer: B

19. What is the current amplification factor α in common-base configuration?

A) IC/IB

B) IE/IB

C) IC/IE

D) IB/IC

Answer: C

20. The typical range of α given in the notes is:

A) 0.01–0.1

B) 0.2–0.5

C) 0.9–0.99

D) 20–500

Answer: C

21. What is the current amplification factor β in common-emitter configuration?

A) IC/IB

B) IE/IC

C) IB/IE

D) IC/IE

Answer: A

22. The typical range of β given in the notes is:

A) 0.9–0.99

B) 1–5

C) 20–500

D) 750–1000

Answer: C

23. Which relation between α and β is correct?

A) β = α/(1−α)

B) β = 1−α

C) β = α+1

D) β = 1/α

Answer: A

24. If α = 0.96, what is β?

A) 12

B) 24

C) 48

D) 96

Answer: B

25. Which BJT configuration is used in about 90–95% of transistor applications according to the notes?

A) Common base

B) Common emitter

C) Common collector

D) Common gate

Answer: B

26. What is the main reason the common-emitter configuration is widely used?

A) Very low current gain

B) High current, voltage and power gain

C) No amplification

D) Only low input resistance

Answer: B

27. The input resistance of a common-base circuit is of the order of:

A) A few ohms

B) A few hundred kΩ

C) Several MΩ

D) 750 kΩ

Answer: A

28. The output resistance of a common-base circuit is of the order of:

A) A few ohms

B) Several tens of kΩ

C) A few Ω

D) 20 Ω

Answer: B

29. The input resistance of a common-emitter circuit is of the order of:

A) A few hundred ohms

B) Several MΩ

C) 750 kΩ

D) 25 Ω

Answer: A

30. The common-collector configuration has approximately:

A) Low input and high output resistance

B) Very high input and very low output resistance

C) Zero input resistance

D) Very high output resistance only

Answer: B

FET / JFET

31. What does FET stand for?

A) Field-Effect Transistor

B) Forward Electron Transistor

C) Field Energy Terminal

D) Frequency Effect Transistor

Answer: A

32. A FET is primarily a:

A) Current-controlled device

B) Voltage-controlled device

C) Temperature-controlled device

D) Power-controlled device

Answer: B

33. Which three terminals are found in a FET?

A) Emitter, Base, Collector

B) Anode, Cathode, Gate

C) Source, Drain, Gate

D) Input, Output, Base

Answer: C

34. A key characteristic of a FET is:

A) Very low input impedance

B) High input impedance

C) High base current

D) Low reliability

Answer: B

35. The JFET controls its channel using a:

A) Forward-biased p-n junction

B) Reverse-biased p-n junction

C) Metal contact

D) Transformer

Answer: B

36. According to the notes, JFET operates in:

A) Enhancement mode only

B) Depletion mode only

C) Both only at zero bias

D) Saturation mode only

Answer: B

37. In an n-channel JFET, the main conduction path is made of:

A) p-type material

B) n-type material

C) oxide

D) metal

Answer: B

38. In an n-channel JFET with VGS = 0 and VDS > 0, which current equality is given?

A) ID = 0

B) ID = IS

C) ID = IG

D) IS = 0

Answer: B

39. For the JFET condition VGS = 0 V, the gate current IG is:

A) Maximum

B) Equal to ID

C) 0 A

D) Negative ID

Answer: C

40. At low VDS, a JFET operates in the:

A) Ohmic region

B) Breakdown region

C) Cut-off region

D) Inverted region

Answer: A

41. At pinch-off, the depletion regions:

A) Disappear completely

B) Nearly meet near the drain

C) Become zero near the source

D) Become forward biased

Answer: B

42. In the JFET saturation region, increasing VDS further generally:

A) Increases ID significantly

B) Does not increase ID

C) Makes IG large

D) Makes the channel wider

Answer: B

43. IDSS is defined as:

A) Minimum gate current

B) Maximum drain current when VGS = 0 V

C) Drain current at VDS = 0

D) Gate-source saturation voltage

Answer: B

44. Which equation is Shockley's equation for a JFET?

A) ID = IDSS(1 − VGS/VP)^2

B) ID = βIB

C) ID = VGS/R

D) ID = IDSS + VGS

Answer: A

45. For a JFET, at VGS = VP, Shockley's equation gives:

A) ID = IDSS

B) ID = IDSS/2

C) ID = 0

D) ID = 1 A

Answer: C

46. For a JFET, at VGS = VP/2, ID is:

A) IDSS

B) IDSS/2

C) IDSS/4

D) 0

Answer: C

MOSFET

47. What does MOSFET stand for?

A) Metal-Oxide-Semiconductor Field-Effect Transistor

B) Metal Output Silicon Field Electronic Transistor

C) Main Oxide Semiconductor Frequency Transistor

D) Metal-Oriented Signal FET

Answer: A

48. Which MOSFET mode is normally OFF at VGS = 0?

A) Depletion mode

B) Enhancement mode

C) Saturation mode

D) Ohmic mode

Answer: B

49. Which MOSFET mode is normally ON at VGS = 0?

A) Enhancement mode

B) Depletion mode

C) Cut-off mode

D) Triode mode

Answer: B

50. Which are the two MOSFET channel types?

A) N-channel and P-channel

B) E-channel and B-channel

C) Source-channel and drain-channel

D) Alpha-channel and beta-channel

Answer: A

51. The gate of a MOSFET is insulated, resulting in:

A) Low input impedance

B) High input impedance

C) High base current

D) Large gate current

Answer: B

52. NMOS stands for:

A) Negative Metal Oxide System

B) N-channel MOSFET

C) New Mode Operating Switch

D) N-type Mechanical Oxide Semiconductor

Answer: B

53. For an enhancement-type NMOS, conduction begins when VGS is:

A) Below VTN

B) Equal to 0 only

C) Greater than or equal to VTN

D) Always negative

Answer: C

54. For an enhancement NMOS in the saturation region, the condition is:

A) VDS ≥ VGS − VTN

B) VDS = 0

C) VGS < VTN

D) VDS < 0

Answer: A

55. For an enhancement NMOS in saturation, the drain current is given by:

A) ID = Kn(VGS − VTN)

B) ID = 1/2 Kn(VGS − VTN)^2

C) ID = VDS/R

D) ID = βIB

Answer: B

56. The MOSFET overdrive voltage is:

A) VOV = VGS + VTN

B) VOV = VGS − VTN

C) VOV = VDS − VGS

D) VOV = VTN − VGS

Answer: B

57. For an enhancement NMOS in the triode/linear region, the condition is:

A) VGS ≥ VTN and 0 ≤ VDS ≤ VGS − VTN

B) VGS < VTN

C) VDS > VGS

D) VGS = 0 only

Answer: A

58. Which device is described in the notes as a foundation of modern digital integrated circuits?

A) JFET

B) MOSFET

C) Vacuum tube

D) Diode

Answer: B

59. In the MOSFET example, if VTN = 1 V, VGS = 3 V and VDS = 4.5 V, the transistor is in:

A) Cut-off

B) Triode only

C) Saturation

D) Breakdown

Answer: C

60. In the MOSFET example, if VTN = 1 V, VGS = 3 V and VDS = 1 V, the drain current is given in the notes as:

A) 0.2 mA

B) 0.6 mA

C) 1.0 mA

D) 2.0 mA

Answer: B

61. For the worked NMOS example with VTN = 0.4 V and Kn = 1.40 mA/V², at VGS = 0.8 V, ID is approximately:

A) 0.024 mA

B) 0.224 mA

C) 1.40 mA

D) 2.02 mA

Answer: B

62. For the same worked NMOS example, at VGS = 1.6 V, ID is approximately:

A) 0.224 mA

B) 0.80 mA

C) 2.02 mA

D) 4.48 mA

Answer: C

Source Basis

BJT lecture: introduction, construction, working regions, α/β, configurations, characteristics, Q-point and transistor states.

Transistor lecture: transistor structure, carrier action, common-base/common-emitter/common-collector configurations, α/β/γ, load-line and switching states.

FET lecture: FET characteristics, JFET operation, pinch-off, saturation, IDSS and Shockley equation.

MOSFET lecture: enhancement/depletion modes, NMOS/PMOS, regions of operation, threshold/overdrive voltage and drain-current equations/examples.